ROBOTIC ARM : EXOGENESIS

Introduction

In recent years, robotic systems have gained importance in medical, industrial, and educational fields. A robotic hand that mimics human finger movements can be useful in rehabilitation, prosthetics, automation, and research. This project focuses on designing and developing a wireless robotic hand using flex sensors and ESP32 microcontrollers. The system captures finger movements using flex sensors mounted on a glove and transmits the data wirelessly to a robotic hand. The robotic hand then replicates the same movements using servo motors.

Objectives of the Project

The main objectives of this project are:

- To design a wearable glove using flex sensors for motion detection.

- To transmit finger movement data wirelessly using ESP-NOW.

- To control servo motors for realistic finger movement.

- To build a low-cost and efficient robotic hand system.

- To ensure reliable and real-time performance.

System Overview

The system is divided into two main parts:

- Sender Unit (Glove Side)

- Receiver Unit (Robotic Hand Side)

1. Sender Unit

The sender unit consists of:

- ESP32 microcontroller

- Flex sensors

- Voltage divider circuit

- Power supply

It reads the resistance of flex sensors and converts it into digital values.

2.Receiver Unit

The receiver unit consists of:

- ESP32 microcontroller

- Servo motors

- Robotic hand structure

- Power supply

It receives wireless data and controls the servo motors accordingly.

Hardware Components

1 ESP32 Microcontroller

- Dual-core processor

- Built-in WiFi and Bluetooth

- Supports ESP-NOW protocol

- Low power consumption

2 Flex Sensors

- Resistance varies with bending

- Used for detecting finger movement

- Connected using voltage divider

3 Servo Motors

- Used to control finger movement

- Angle range: 0° to 180°

- Controlled using PWM signals

Software Requirements

- Arduino IDE

- ESP32 Board Package

- ESP32Servo Library

- ESP-NOW Library

Working Principle

- Flex sensors change resistance when bent.

- ESP32 reads analog values using ADC.

- Resistance is calculated using voltage divider formula.

- Noise is reduced using smoothing techniques.

- Finger states (bent/flat) are determined using hysteresis logic.

- Data is transmitted using ESP-NOW.

- Receiver ESP32 receives data.

- Servo motors rotate according to received values.

- Robotic hand mimics human hand movement.

Code Implementation and Hardware Setup

1 Sender (Glove Unit) Code

The sender unit reads flex sensor values, processes them, and transmits data wirelessly using ESP-NOW.

Main Functions:

- Reads analog values from flex sensors

- Applies smoothing and hysteresis

- Calculates resistance

- Sends finger states to receiver

(Refer to project source code for complete implementation.)

2 Receiver (Robotic Hand Unit) Code

The receiver unit receives data and controls servo motors accordingly.

Main Functions:

- Initializes ESP-NOW

- Receives finger state data

- Controls servo motors using PWM

- Implements safety timeout

(Refer to project source code for complete implementation.)

3 Software Setup Procedure

- Install Arduino IDE.

- Install ESP32 board package.

- Install ESP32Servo library.

- Connect ESP32 to computer.

- Select correct COM port and board.

- Upload receiver code first.

- Note down MAC address.

- Update sender code with receiver MAC.

- Upload sender code.

- Open Serial Monitor for debugging.

4 Hardware Connections

Flex Sensor Connections (Sender)

- One end of flex sensor → 3.3V

- Other end → Analog Pin (GPIO)

- Fixed resistor (200kΩ) → Ground

- Voltage divider output → ESP32 ADC pin

Servo Motor Connections (Receiver)

- Red wire → 5V external supply

- Brown/Black wire → Ground

- Yellow/White wire → ESP32 PWM pin

- Common ground between ESP32 and power supply

Power Supply

- ESP32 powered via USB or 5V regulator

- Servos powered using external 5V supply

5 System Setup Steps

- Mount flex sensors on glove fingers.

- Fix servo motors on robotic hand structure.

- Connect all sensors and motors.

- Verify power connections.

- Upload codes.

- Test individual finger movement.

- Calibrate resistance values.

- Perform wireless testing.

- Final system integration.

Flowchart

Communication Method (ESP-NOW)

ESP-NOW is a low-latency wireless communication protocol developed by Espressif. It enables peer-to-peer communication without using a router.

Features:

- Low delay

- Low power

- High reliability

- No internet required

Algorithms Used

1 Smoothing Algorithm

Moving average filter is used to reduce noise in sensor readings.

2 Hysteresis Algorithm

Two thresholds (R_LOW and R_HIGH) are used to prevent frequent switching.

3 Servo Control Algorithm

Finger states are mapped to servo angles for realistic movement.

Results and Observations

- The robotic hand accurately follows human finger movements.

- Wireless transmission is stable within 20–30 meters.

- System response time is less than 50 ms.

- Power consumption is moderate.

- System is reliable for continuous operation.

Applications

- Prosthetic hand control

- Rehabilitation systems

- Human–machine interaction

- Robotics research

- Educational projects

Advantages

- Low cost

- Wireless operation

- Real-time control

- Portable

- Easy to modify

Limitations

- Limited range

- Requires calibration

- Sensitive to noise

Future Scope

- Mobile app control

- AI-based gesture recognition

- Haptic feedback

- Cloud monitoring

References

- ESP32 Datasheet – Espressif Systems

- Arduino Official Documentation

- ESP-NOW Programming Guide

- Robotics and Sensor Technology Books

- ESP32 Datasheet – Espressif Systems

- Arduino Official Documentation

- ESP-NOW Programming Guide

- Robotics and Sensor Technology Books

Conclusion

This project successfully demonstrates the design and implementation of a wireless robotic hand using ESP32 and flex sensors. The system is efficient, low-cost, and suitable for academic and research purposes. It provides a strong foundation for advanced robotic and prosthetic applications.

YOUTUBE VIDEOS LINK TESTING AND PROTOTYPE

- https://youtube.com/shorts/7Oa2iwueIwg?si=Fzh45fDLNvpiwW9M

- https://youtube.com/shorts/HdQvleQe6Do?si=t_su_lGh6K9XxUNW

LINKEDIN POST

- https://www.linkedin.com/posts/harshvardhanshah63_robotics-embeddedsystems-hardwareprojects-activity-7410461919540752384-7Hrv?utm_source=social_share_send&utm_medium=android_app&rcm=ACoAAF4nhrcB4VpFJhVOthZDY2YUtMuMLE54Vss&utm_campaign=copy_link

- https://www.linkedin.com/posts/harshvardhanshah63_roboticsdemo-mechatronics-esp32-activity-7410465693500022785-tMFq?utm_source=social_share_send&utm_medium=android_app&rcm=ACoAAF4nhrcB4VpFJhVOthZDY2YUtMuMLE54Vss&utm_campaign=copy_link

GITHUB REPOSITORY

https://github.com/harshvardhanshah63/Robotic-Arm-Esp-Now-Flex-Sensor-

PRODUCT IMAGES